SCM DELIVERY PROCEDURE

AccessControlList — Group 1

CSCI 234  |  Moravian University  |  Spring 2025

| Field | Value |
| --- | --- |
| Document ID | DP-ACL-2025-001 |
| Version | 1.0 |
| Date | May 2025 |
| Author / Owner | Mohammad (SCM Manager) |
| Status | Released — Baseline B1.0 |
| Target Hardware | Raspberry Pi 3 (RP3) + BeagleBone Black (BBB) |
| Software | AccessControlList — Group 1 (Java 21) |

# 1. Scope and Purpose

This document provides a step-by-step Software Configuration Management (SCM) Delivery Procedure for the AccessControlList (ACL) subsystem developed by Group 1 as part of the PWMS (Predictive Wastewater Management System) project. The procedure begins with loading the Raspberry Pi 3 (RP3) Operating System onto a formatted SD card and concludes with the RP3 running and connected to the BeagleBone Black (BBB) host computer, with the ACL server software active and ready for test.

# 2. Prerequisites and Required Materials

## 2.1 Hardware

| Item | Specification / Notes |
| --- | --- |
| Raspberry Pi 3 Model B | Target server node running the ACL software |
| BeagleBone Black (BBB) | Preferred host/client node; a standard laptop may substitute |
| MicroSD Card | 16 GB or larger, Class 10 or better |
| SD Card Reader/Writer | USB adapter acceptable |
| Ethernet Cable (x2) | Cat 5e or better — RP3 to router/switch, BBB to router/switch |
| Network Switch or Router | Both RP3 and BBB on the same local subnet |
| 5V / 2.5A Micro-USB Power Supply | For RP3 |
| 5V / 1A DC Barrel Supply | For BBB |
| HDMI Monitor + Cable | For initial RP3 configuration only |
| USB Keyboard + Mouse | For initial RP3 configuration only |

## 2.2 Software

| Item | Version / Source |
| --- | --- |
| Raspberry Pi OS (64-bit, Lite or Desktop) | Latest from https://www.raspberrypi.com/software/operating-systems/ |
| Raspberry Pi Imager | Latest from https://www.raspberrypi.com/software/ |
| Java JDK 21 | OpenJDK 21 — installed on RP3 in Step 5 |
| Git | Installed on RP3 in Step 5 |
| AccessControlList source code (Baseline B1.0) | https://github.com/your-org/AccessControlListGroup1 (tag B1.0) |
| Java JDK 21 (BBB) | Installed on BBB in Step 8 |

## 2.3 Configuration Reference

The ACL server reads a configuration file (ID.Config) at startup. The file must be present in the working directory. The default content for a single-node test where both the TST client and ACS server share one IP address is:

TST TST <RP3_IP> 1002 ACS <RP3_IP> 1002

NOTE: Replace <RP3_IP> with the actual IP address discovered in Step 4. When the BBB acts as the TST node, replace the TST IP with the BBB's address.

# 3. Safety and Return Obligations

IMPORTANT — Per course policy, failure to return the RP3 and all accessories (power supply, SD card, cables) at the end of the course will result in the entire team receiving Incompletes. The SCM Manager (Mohammad) is responsible for ensuring all hardware is collected from team members and returned intact.

# 4. Delivery Procedure

## Phase A — Prepare the Raspberry Pi 3 SD Card

- Format the SD card

- Insert the MicroSD card into your card reader and connect it to your workstation.

- Open Raspberry Pi Imager (download from https://www.raspberrypi.com/software/ if not installed).

- Click CHOOSE OS → Raspberry Pi OS (64-bit) — select Lite for headless or Desktop if a GUI is preferred.

- Click CHOOSE STORAGE and select your MicroSD card. Verify the device name carefully to avoid overwriting the wrong drive.

- Click the gear icon (Advanced Options) and configure: hostname (e.g., acl-server), enable SSH, set username/password, configure Wi-Fi (optional), and set locale.

- Click WRITE. Accept the warning that all data on the card will be erased.

- Wait for the write and verification to complete (~5–10 minutes depending on card speed).

- Boot the Raspberry Pi 3 for the first time

- Remove the MicroSD card from the reader and insert it firmly into the RP3's MicroSD slot (underside of the board).

- Connect the HDMI monitor and USB keyboard/mouse to the RP3.

- Connect the Ethernet cable from the RP3 to the network switch or router.

- Plug in the 5V/2.5A Micro-USB power supply. The RP3 will boot automatically (no power button).

- Wait for the green activity LED to settle (~60 seconds for first boot). If using Raspberry Pi OS Desktop, complete the initial setup wizard.

- If using the Lite (headless) image and SSH was configured in Advanced Options, you may connect via SSH from your workstation instead of using the monitor/keyboard.

- Complete initial OS configuration

- Open a Terminal on the RP3 (or SSH into it from your workstation).

- Update the package lists and upgrade installed packages:

sudo apt-get update sudo apt-get upgrade -y

- Set the system timezone if not already correct:

sudo raspi-config # Navigate to: Localisation Options -> Timezone

- Reboot after upgrades:

sudo reboot

- Identify the RP3 IP address

- After reboot, open a terminal on the RP3 (or reconnect via SSH).

- Run the following command and record the IP address of the eth0 interface:

hostname -I

- Example output: 10.180.137.217. Note this address — it is used in the ID.Config file.

NOTE: If using Wi-Fi, the wlan0 address appears alongside the eth0 address. Use the eth0 (wired) address for reliability.

## Phase B — Install Java and the ACL Software on RP3

- Install Java JDK 21 and Git on the RP3

- In a terminal on the RP3, install the required packages:

sudo apt-get install -y git openjdk-21-jdk

- Verify the Java installation:

java -version # Expected output: openjdk version "21.x.x" ... javac -version # Expected output: javac 21.x.x

- Clone the baseline B1.0 source code

- Navigate to the home directory and clone the repository at the B1.0 baseline tag:

cd ~ git clone https://github.com/your-org/AccessControlListGroup1.git cd AccessControlListGroup1 git checkout B1.0

- Confirm the correct baseline is checked out:

git log --oneline -1 # Should display the commit tagged B1.0

NOTE: If internet access is unavailable on the RP3, copy the zip file to the RP3 via SCP from your workstation: scp AccessControlListGroup1-main.zip pi@<RP3_IP>:~ and then unzip it.

- Create and configure the ID.Config file

- Navigate to the src source directory:

cd ~/AccessControlListGroup1/src/Transport/

- Open ID.Config in a text editor and set the IP addresses. For a run where the RP3 is both server and TST client (single-node test):

nano ID.Config # File contents — replace 10.180.137.217 with actual RP3 IP: TST TST 10.180.137.217 1002 ACS 10.180.137.217 1002

- For a two-node test where the BBB acts as the TST client, use the BBB IP for the TST entry and the RP3 IP for the ACS entry:

TST TST <BBB_IP> 1002 ACS <RP3_IP> 1002

- Save the file (Ctrl+O, Enter, Ctrl+X in nano).

- Compile the ACL software on the RP3

- Navigate to the project root and compile all source files:

cd ~/AccessControlListGroup1 mkdir -p out javac -d out -sourcepath src \ src/Presentation/SubsystemEnums.java \ src/Presentation/SubsystemRoles.java \ src/Presentation/AASInterface.java \ src/Presentation/ACSInterface.java \ src/Presentation/AASConnector.java \ src/Presentation/ACSConnector.java \ src/Presentation/AASBoundary.java \ src/Presentation/ACSBoundary.java \ src/Application/AccessControlList.java \ src/Application/Authenticate.java \ src/Transport/ConfigFile.java \ src/Transport/RunTimeVars.java \ src/Transport/P2PServer.java \ src/Transport/P2PClient.java \ src/Main.java

- If compilation succeeds, no output is produced and the out/ directory will contain the compiled .class files. Verify:

ls out/Presentation/ out/Transport/ out/Application/

NOTE: If the compiler reports errors related to com.sun.security.auth.login (imported in RunTimeVars.java), this import is unused; remove it or ignore the warning — it does not prevent compilation with OpenJDK 21.

## Phase C — Connect the BeagleBone Black (BBB)

- Prepare the BeagleBone Black

- Connect the BBB to the same network switch or router as the RP3 using an Ethernet cable.

- Power on the BBB via its 5V DC barrel connector.

- Open a terminal on the BBB (or SSH into it from your workstation).

- Verify the BBB can reach the RP3 by pinging it:

ping -c 4 <RP3_IP>

- Verify the RP3 can reach the BBB:

ping -c 4 <BBB_IP> # run on the RP3 terminal

NOTE: Both ping commands must succeed before proceeding. If ping fails, check cable connections, verify both devices are on the same subnet, and check firewall settings.

- Install Java JDK 21 on the BeagleBone Black

- On the BBB terminal, install Java:

sudo apt-get update sudo apt-get install -y openjdk-21-jdk java -version # Expected: openjdk version "21.x.x"

- Deploy the compiled ACL classes to the BBB (optional for TST role)

- If the BBB will run the TST (test) client, copy the compiled output from the RP3:

# Run on the BBB: scp -r pi@<RP3_IP>:~/AccessControlListGroup1/out ~/AccessControlListGroup1-out scp pi@<RP3_IP>:~/AccessControlListGroup1/src/Transport/ID.Config ~/

- Alternatively, clone and compile the repository directly on the BBB using the same commands as Step 6–8.

## Phase D — Start the ACL Server and Verify Connectivity

- Start the ACL server on the RP3

- On the RP3 terminal, run the main server program from the project root. The server will begin listening on port 1002:

cd ~/AccessControlListGroup1 java -cp out Main

- Expected console output (from ServerTransactionLogger):

Server Created. Port: 1002 Server Started Connecting -- waiting for client on port 1002

NOTE: The server blocks waiting for a client connection. Leave this terminal open. It will log each accepted connection and authentication result.

- Run the TST client on the BBB (or RP3) to verify the connection

- Open a new terminal on the BBB (or a second terminal on the RP3 for a single-node test).

- Start the client, which will connect to the server at port 1002 and present the test menu:

cd ~/AccessControlListGroup1-out # or ~/AccessControlListGroup1 if compiled locally java -cp out Main

- The CLI will display subsystem/role options. Select an authentication scenario (e.g., Data Analyst Log In).

- Observe the RP3 server console: it will print Accepted client on port 1002, the message type received, and the authorization result (Authorized / NOT Authorized).

- Compare the output against the expected results in the Server Output Log (ServerOutputLog-From-Voyager-2.txt) included in the repository.

NOTE: A successful delivery is confirmed when the server logs match the expected pattern: Accepted client, Server received Authenticate, and either Authorized or NOT Authorized per the test case.

- Verify the ACS.dat persistent data store

- When the server processes its first authentication request, it creates (or reads) ACS.dat in the working directory. Verify it exists:

ls -lh ~/AccessControlListGroup1/ACS.dat

- If the file does not exist, the AccessControlList singleton will create a new one with a default SysAdmin record on first access.

## Phase E — Final Verification Checklist

| # | Verification Item | Expected Result | Pass / Fail |
| --- | --- | --- | --- |
| 1 | RP3 boots from SD card | RP3 reaches login prompt / desktop |  |
| 2 | Network connectivity | RP3 and BBB ping each other successfully |  |
| 3 | Java 21 on RP3 | java -version reports OpenJDK 21 |  |
| 4 | Java 21 on BBB | java -version reports OpenJDK 21 |  |
| 5 | ID.Config has correct IPs | File contains RP3/BBB IPs on correct lines |  |
| 6 | Source compiles without errors | javac produces no error output; .class files present in out/ |  |
| 7 | Server starts and listens | Console shows Server Started, Connecting -- waiting for client on port 1002 |  |
| 8 | Client connects to server | Server logs Accepted client on port 1002 |  |
| 9 | Auth request processed | Server logs Authorized or NOT Authorized per test case |  |
| 10 | ACS.dat persists across restarts | Data store retains added users after server restart |  |

# 5. Rollback Procedure

If the deployed software fails to operate correctly and cannot be quickly corrected, revert to the last known-good baseline using the following steps:

# On the RP3: cd ~/AccessControlListGroup1 git fetch --tags git checkout B1.0 # Re-compile as in Step 8, then restart the server as in Step 12.

# 6. Configuration Items Under SCM Control

| CI Name | Type | Baseline | Location |
| --- | --- | --- | --- |
| AccessControlList.java | Source code | B1.0 | src/Application/ |
| Authenticate.java | Source code | B1.0 | src/Application/ |
| ACSConnector.java | Source code | B1.0 | src/Presentation/ |
| AASConnector.java | Source code | B1.0 | src/Presentation/ |
| ACSBoundary.java | Source code | B1.0 | src/Presentation/ |
| AASBoundary.java | Source code | B1.0 | src/Presentation/ |
| SubsystemEnums.java | Source code | B1.0 | src/Presentation/ |
| SubsystemRoles.java | Source code | B1.0 | src/Presentation/ |
| Main.java | Source code | B1.0 | src/ |
| ID.Config | Configuration file | B1.0 | src/Transport/ |
| ACS.dat | Runtime data store | N/A (generated) | Working directory on RP3 |
| DP-ACL-2025-001 (this document) | SCM document | B1.0 | Project repository /docs/ |
| CMP-ACL-2025-001 | CM Plan | B1.0 | Project repository /docs/ |
| BL-ACL-2025-001 | Baseline log | B1.0 | Project repository /docs/ |

# 7. Document Revision History

| Version | Date | Author | Description |
| --- | --- | --- | --- |
| 1.0 | May 2025 | Mohammad (SCM Manager) | Initial release — Baseline B1.0 |

End of Document — DP-ACL-2025-001 v1.0